Circular RSA VI 02/08

8th February 2008.

To all Authorised Officers and Authorised Test Centre’s

Item number 59 of the Heavy Commercial Vehicle Testers Manual 2004 outlines the Methods of Testing and Reasons for Failure for Side Lamps and Marker Lamps. The Method of Testing for a Marker Lamp is as follows;

“Check that a marker lamp on a trailer / semi-trailer is fitted midway on the right hand side and the illuminated surface of the lamp shows a white light and is facing outwards and to the front at an angle of 45 degrees to the longitudinal axis of the trailer / semi-trailer. The illuminated surface should not be higher than 1.52m (5’) from the ground. The marker lamp requirement applies to trailers or semi-trailers in combinations exceeding 12.2m (40’) in length.”

As of today, EC specification Side Marker Lamps will be accepted as an alternative to the Marker Lamp test requirements as described in the Test Manual. The Method of Testing for EC specification Side Marker Lamps is as follows;

“On each side of the vehicle, check that at least one side-marker lamp is fitted to the middle third of the vehicle, the foremost side-marker lamp being not further than 3m from the front and distance between two adjacent side-marker lamps not exceeding 3m (if the structure of the vehicle makes it impossible to comply with such a requirement, this distance may be increased to 4m). The distance between the rearmost side-marker lamp and the rear of the vehicle shall not exceed 1m. The lamps should be positioned not less than 350 mm and not more than 1,500 mm from the ground (if the structure of the vehicle does not allow for compliance with the maximum height, this limit may be raised to 2,100mm). The side-marker lamps should be amber in colour (the rearmost side-marker lamp may be red if grouped or combined or reciprocally incorporated with the rear position lamp) and should be visible from a 45 degree angle from the front and rear of the vehicle. The marker lamp requirement applies to trailers or semi-trailers in combinations exceeding 12.2m (40’) in length.”

The Reasons for Failure for EC specification Side Marker Lamps are as follows;

- Missing

- Not working or faulty

- Badly cracked or missing lens

- Incorrect colour

- Incorrectly positioned

It is a vital for road safety that trailers / semi-trailers can be clearly indentified to oncoming traffic during hours of darkness and these Marker Lamps fulfill this function. It is essential that a trailer or semi-trailer does not pass the roadworthiness test if it does not display either of these two Marker Lamp options. Please contact the undersigned if any clarification is required in this matter.

_____________________

Seán Breathnach

Senior Vehicle Inspector

Tel: 091 872629.